Micro-Project (Part-1)

For partial fulfillment of Activity Based Learning

for Course

Data Communication & Networking (3028)

Network Design Task-01

Submitted By:

Arpita Srivastava (2104069)

Navanita Das (2104111)

Saurav Kr. Yadav (2104117)

Rashmita Biswal (2104118)

School of Electronics Engineering

Kalinga Institute of Industrial Technology (KIIT)

Deemed to be University, Bhubaneswar -751024

Aim :

Design, configuration and simulation of heterogeneous network i.e. wired (IEEE 802.3, ETHERNET) and wireless (IEEE 802.11, WLAN) with two routers along with the use of CISCO networking devices for the analysis of traffic in network and other observations using CISCO Packet Tracer.

Network  Scenario :

Software used:

CISCO Packet Tracer

Network  Specifications :

- Heterogeneous Network

Table. 1

| Name | Network Address | IP Configurations | Gateway |
| --- | --- | --- | --- |
| OFFICE_1 | 192.168.1.0/24 | 192.168.1.2/24 - 192.168.1.254/24 | 192.168.1.1/24 |
| OFFICE_2 | 192.168.2.0/24 | 192.168.2.2/24 - 192.168.2.254/24 | 192.168.2.1/24 |
| INTERNET | 10.0.0.0/8 | 10.0.0.2/8 - 10.255.255.254/8 | 10.0.0.1/8 |

* IP Configuration: Dynamic Host Configuration Protocol (DHCP)

Note: All are Class-C Networks default subnet mask: 255.255.255.0

- Internet (Network Address : 10.0.0.0/8 )

- Class-A Networks default subnet mask: 255.0.0.0

HTTP(s) Server hosted on a Generic Server (End Device).

IP Address of Server machine 10.0.0.2 (e.g www.kiit.ac.in)

Default Gateway: 10.0.0.1

- Cable Specifications

  - Connections > Copper Straight -Through

(Between PC/Laptop/Server to Switch and Switch to Routers)

  - Connections > Copper Cross – Over

- (Between same device like Switch to Switch and Routers to Routers)

- Switch Specifications

  - Type: CISCO 2950T-24 (Switch for Wired LAN)

  - IEEE 802.3 Fast Ethernet (FE-Copper)

  - Standard: 100-Base_TX

- Wireless Access Points / Routers

    - Type: Linksys-WRT300N

    - IEEE 802.11 standard - ISM Band 2.4 GHz

    - Authentication: WPA2-PSK

    - Encryption: AES

    - SSID: OFFICE_2 for respective office location

    - Pass Phrase: dcn@etc2

    - LAN: IP Configuration: DHCP as per Table.1

- Router Specifications

  - Type: Generic (Router-PT-Empty)

  - Add Hardware interfaces: Fast Ethernet 100 Mbps

  - Network Interface Card (NIC) – Network Adaptor : IEEE 802.3 Fast Ethernet (FE-Copper)

  - Standard: 100-Base_TX

  - 3 NICs for Router 1 (R1)

  - 2 NICs for Router 2 (R2)

  - Routing Protocol: Routing Information Protocol v.1 (RIP v1)

- Intermediate Network  Specifications :

| Between Routers | Network Address | IP Address of Gateways |
| --- | --- | --- |
| AP (OFFICE_2) – R2 | 192.168.3.0/24 | 192.168.3.1 & 192.168.3.2 |
| R1- R2 | 192.168.4.0/24 | 192.168.4.1 & 192.168.4.2 |

NB: AP             Access Point (Linksys-WRT300N)

PC/ Laptop/ Server Specifications :

  - End Devices > Generic PC/laptop/Server

Procedure:

As per the Network scenario diagram given above, the required networking devices like Wireless Access Points, Routers, Switches, PCs, Laptops, Wireless Tablets and Smart devices (PDA) were placed. The required cable connections were made. All devices were configured as per  the  specifications given above. Some of the device configuration methods (Screenshots) are given below:

Basic Configuration

- Wireless Access Point Configuration (Linksys-WRT300N) Config > Wireless

Config > Internet

GUI (LAN DHCP Configuration)

Then, Save Settings.

- Laptop

“Linksys-WPC300N” module is inserted into laptop and then it is switched on.

Config > laptop

- Wireless Tablet/ Smart device (PDA)

- Other wired end devices

All PC connected to wired Ethernet Network must be configured as

Desktop > IP Configuration > DHCP.

- DHCP Server

- Router

- Place Generic Routers from the Router Menu from the Lower left corner of the window.

- Double-Click on Router to open Menu. In PHYSICAL TAB.Add a extra PT-ROUTER NM-1CFE interface by dragging and dropping at the back panel of the Router.

- Similar to placement of Router, place CISCO Catalyst 2950T-24 Switches and End devices like PC and Server as required.

- Connect Copper Straight Cable between PC-SWITCH, SWITCH-ROUTER.

- Connect Copper Cross Cable between ROUTER-ROUTER and SWITCH-SWICTH interfaces.  Give the suitable IP Address in Router interface by refer by double clicking on ROUTER > COFIG > Select the suitable interface> Give the IP Address in the Space given.

- The above mentioned steps are shown in the figures below:

R1

---------------------------------------------------------------------------

R2

- Routing Protocol: RIPv1

- HTTP Server

Observations:

After designing and configuring the complete network,  followings can verified and observered:

- Wireless devices getting associated with the Assess Point using Radio Links.

- DHCP  messages for automatic assignment of IP Configuration.

- PING Command

PC > Desktop > Command Prompt:

Ping 192.168.2.101

- HTTP over TCP Traffic in the Network in Simulation mode

- PC > Desktop > Web Browser :

- Complex PDU

*** END OF REPORT ***